module alu_4bit (

input  wire [3:0] a,

input  wire [3:0] b,

input  wire [1:0] op,

input  wire       clk,

input  wire       rst_n,

output wire [3:0] result,

output wire       zero

);

reg [3:0] result_reg;

// Sequential ALU with asynchronous active-low reset

always @(posedge clk or negedge rst_n) begin

if (!rst_n)

result_reg <= 4'b0000;

else begin

case (op)

2'b00: result_reg <= a + b;

2'b01: result_reg <= a - b;

2'b10: result_reg <= a & b;

2'b11: result_reg <= a | b;

default: result_reg <= 4'b0000;

endcase

end

end

assign result = result_reg;

assign zero   = (result_reg == 4'b0000);

endmodule

Testbench :-

`timescale 1ns/1ps

module tb;

reg [3:0] a, b;

reg [1:0] op;

reg clk, rst_n;

wire [3:0] result;

wire zero;

integer i;

integer passed;

integer failed;

// DUT: Device Under Test

alu_4bit dut (

.a(a),

.b(b),

.op(op),

.clk(clk),

.rst_n(rst_n),

.result(result),

.zero(zero)

);

// Clock: 10 ns period

initial clk = 1'b0;

always #5 clk = ~clk;

// Task to check one ALU operation

task run_test;

input [3:0] a_in;

input [3:0] b_in;

input [1:0] op_in;

reg [3:0] expected;

begin

// Calculate expected result

case (op_in)

2'b00: expected = a_in + b_in;

2'b01: expected = a_in - b_in;

2'b10: expected = a_in & b_in;

2'b11: expected = a_in | b_in;

default: expected = 4'b0000;

endcase

// Apply inputs away from the active clock edge

@(negedge clk);

a  = a_in;

b  = b_in;

op = op_in;

// Wait for the register to update

@(posedge clk);

#1;

// Check result and zero flag

if ((result !== expected) ||

(zero !== (expected == 4'b0000))) begin

$display(

"FAIL: a=%0d b=%0d op=%b expected=%0d result=%0d zero=%b",

a_in, b_in, op_in, expected, result, zero

);

failed = failed + 1;

end

else begin

$display(

"PASS: a=%0d b=%0d op=%b result=%0d zero=%b",

a_in, b_in, op_in, result, zero

);

passed = passed + 1;

end

end

endtask

initial begin

a = 0;

b = 0;

op = 0;

rst_n = 0;

passed = 0;

failed = 0;

$dumpfile("dump.vcd");

$dumpvars(0, tb);

$display("--------------------------------------");

$display(" 4-BIT ALU FUNCTIONAL VERIFICATION");

$display("--------------------------------------");

// Check asynchronous reset

#1;

if ((result !== 4'b0000) || (zero !== 1'b1)) begin

$display("FAIL: Initial asynchronous reset");

failed = failed + 1;

end

else begin

$display("PASS: Initial asynchronous reset");

passed = passed + 1;

end

@(negedge clk);

rst_n = 1'b1;

// Test all four operations

run_test(4'd7,  4'd3,  2'b00); // ADD = 10

run_test(4'd8,  4'd5,  2'b01); // SUB = 3

run_test(4'd15, 4'd7,  2'b10); // AND = 7

run_test(4'd0,  4'd0,  2'b11); // OR = 0, zero flag

// Boundary and overflow cases

run_test(4'd15, 4'd1,  2'b00); // 15+1 = 0 (4-bit)

run_test(4'd2,  4'd5,  2'b01); // 2-5 = 13 (4-bit)

run_test(4'd0,  4'd15, 2'b10); // AND = 0

run_test(4'd8,  4'd1,  2'b11); // OR = 9

// Check asynchronous reset during operation

@(negedge clk);

a = 4'd9;

b = 4'd6;

op = 2'b00;

@(posedge clk);

#1;

#1 rst_n = 1'b0;

#1;

if ((result !== 4'b0000) || (zero !== 1'b1)) begin

$display("FAIL: Asynchronous reset during operation");

failed = failed + 1;

end

else begin

$display("PASS: Asynchronous reset during operation");

passed = passed + 1;

end

@(negedge clk);

rst_n = 1'b1;

// Randomized testing

for (i = 0; i < 20; i = i + 1) begin

run_test(

$urandom_range(0, 15),

$urandom_range(0, 15),

$urandom_range(0, 3)

);

end

// Final summary

$display("--------------------------------------");

$display("TOTAL PASSED = %0d", passed);

$display("TOTAL FAILED = %0d", failed);

$display("--------------------------------------");

if (failed == 0)

$display("ALL FUNCTIONAL TESTS PASSED");

else

$display("SOME TESTS FAILED");

$finish;

end

endmodule